Linux Initrd Ram Disk Image for Booting with The Root Filesystem on A PCMCIA device

Version 0.2, December 9, 2003

Copyright © 2003 VIA Technologies, INC.

- Summary

The document describes how to build an initrd ram disk image for booting with the root filesystem on a pcmcia device. The normal linux kernel booting sequence initials PCMCIA devices after the root device mounted. We need to initial the PCMCIA device before the kernel tries to mount the root filesystem on the PCMCIA device. The information in this document is provided “AS IS” without guarantee of any kind.

- Rebuilding kernel

For using initrd ram disk in the booting sequence, we must make sure the kernel supports initrd ram disk. If the kernel doesn’t support that we have to rebuild kernel and reboot the rebuilt kernel to make ram disk image.

    - Type “make menuconfig” in the kernel source directory and enter the “Block devices”.

    - Select “RAM disk support” for built-in and you can change the supportable ram disk size, and then select “Initial RAM disk (initrd) support”.

    - Back to the top menu and enter the “General setup” and enter “PCMCIA/CardBus support”. Then select “PCMCIA/CardBus support” for module and select “CardBus support” and “i82365 compatible bridge support”.

    - Back to the top menu and enter the “ATA/IDE/MFM/RLL support”. In the “ATA, IDE and ATAPI Block devices” entry, select “PCMCIA IDE support” for module.

    - Select other modules you need, save the configuration and type “make dep clean bzImage modules modules_install” to build your kernel and modules.

- Create initrd ram disk image

After rebuilding kernel, reboot machine with new kernel. The following shows the command we do for creating initrd ram disk image.

# pcinitrd –v initrd pcmcia/yenta_socket.o	create image

# mount –o loop –t ext2 initrd /tmp			mount image using the

loop back device

# cp /lib/modules/2.4.18-14custom/kernel/drivers/ide/ide-cs.o /tmp/lib/pcmcia							copy necessary driver

# vi /tmp/linuxrc							customize startup sequence

Change “PCIC=i82365” to “PCIC=yenta_socket” and add “insmod $PC/ide-cs.o” under “insmod $PC/ds.o”.

# umount /tmp								umount the loop back device

# mount /dev/hde1 /mnt						mount compact flash

# gzip < initrd > /mnt	/initrd				gzip the initrd image

- Booting the root filesystem with a PCMCIA device

Now we have the new kernel and the initrd ram disk image. We must put them on the boot device, and then command loader to load initrd ram disk image by adding parameter “initrd=/directory/initrd” and specify root device with parameter “root=/dev/hd[e-t][1-32]” in “/etc/lilo.conf”.

- Test configuration

The system configurations used for this test are described below.

| CPU | VIA C3-2 1GHz (133*7) |
| --- | --- |
| MB | EPIA-M2 10000 (CLE266 + VT8235) |
| RAM | 128MB DDR 266 DRAM |
| HDD | Maxtor D740X-6L 40 GB |
| OS (kernel) | RedHat Linux 8.0 (2.4.18-14) |
| Compact Flash | Apacer 64 MB |