Linux Kernel Modification for Booting with The Root Filesystem on An USB 2.0 Device

Version 0.1, April 12, 2004

Copyright © 2004 VIA Technologies, INC.

- Summary

The document describes how to modify the Linux kernel source for booting with the root filesystem on an USB 2.0 device. The Linux kernel needs to execute the EHCI driver and the mass storage device driver before it tries to mount the root filesystem on the USB 2.0 device. The information in this document is provided “AS IS” without guarantee of any kind.

- Modify the Linux kernel source

The following shows how to modify the Linux kernel souce.

# cd /usr/src/linux-2.4/init/				enter the target directory

# vi do_mounts.c							edit the target file

In the mount_block_root function,

...............................................

retry:

...............................................

/*

* Allow the user to distinguish between failed open

* and bad superblock on root device.

*/

You can change the message output by printk function.

printk ("Please append a correct \"root=\" boot option\n");

Comment the panic function.

//panic("VFS: Unable to mount root fs on %s", kdevname(ROOT_DEV));

Add the following three statements for waiting the USB 2.0 device becomes ready.

set_current_state(TASK_INTERRUPTIBLE);

schedule_timeout(HZ);

goto retry;

}

panic("VFS: Unable to mount root fs on %s", kdevname(ROOT_DEV));

out:

- Rebuilding kernel

For booting Linux from USB 2.0 devices, we must make sure the EHCI driver and the mass storage driver were built in the Linux kernel. If not, we have to rebuild the Linux kernel.

    - Type “make menuconfig” in the kernel source directory and enter the “SCSI support”.

    - Select “SCSI support” and “SCSI disk support“ for built-in.

    - Back to the top menu and enter the “USB support”. Then select “Support for USB”, “EHCI HCD (USB 2.0) support (EXPERIMENTAL)”, and “USB Mass Storage support” for built-in.

    - Select other modules you need, and then save the configuration and type ” make dep clean bzImage modules modules_install” to build your kernel and modules.

- Test configuration

The system configurations used for this test are described below.

| CPU | VIA C3-2 1GHz (133*7) |
| --- | --- |
| MB | EPIA-M2 10000 (CLE266 + VT8235) |
| RAM | 128MB DDR 266 DRAM |
| HDD | Maxtor D740X-6L 40 GB |
| OS (kernel) | RedHat Linux 8.0 (2.4.18-14) |
| USB 2.0 Flash | Transcend JetFlash 128MB |