4.3 Default RS-232C Settings

Baud Rate	9600

Stop bit		1

Data bits	8

Parity		None

If users are writing their own applications, they must make sure the DTR must provide 10V (or higher), and the RTS must provide -10V (or lower).  Voltage (10V, -10V) from DTR and RTS is used to generated RS-232C signal.  DTR is usually enabled (10V).  But RTS has to be disabled in order to be -10V.  To do that in BASIC, adding RS parameter in the OPEN command:

OPEN "COM1: 9600, N, 8, 1, CS, DS, CD, RS" as #1

Refer to section 7.4 and 5.4.5 for details if users need to change the baud rate.